Which of the following is contained in Process Control Block (PCB)? [1m]

a. Process id

b. List of Open files

c. Memory Limits

d. Register (SP, PC, general purpose) values

Which of the following state transitions do not appear in a conventional scheduler:  [1m]

(A) running->blocked (B) running->ready (C) blocked->ready (D) blocked->running.

Consider a simple linux shell implementing the command sleep 100. Which of the following is an accurate ordered list of system calls invoked by the shell from the time the user enters this command to the time the shell comes back and asks the user for the next input? [1.5 m]

A. wait-exec-fork

B. exec-wait-fork

C. fork-exec-wait

D. wait-fork-exec

What is/are the possible output(s) printed from this program shown below? You may assume that the program runs on a modern Linux-like OS. You may ignore any output generated from “some executable”.  [1.5m]

int ret = fork();

if(ret == 0) {

printf(‘‘Hello1\n’’);

exec(‘‘some_executable’’);

printf(‘‘Hello2\n’’);

} else if(ret > 0) {

printf(‘‘Hello3\n’’);

} else {

printf(‘‘Hello4\n’’);

}

- Hello1, Hello3 are printed

- Hello1, Hello2, Hello3 are printed.

- Hello4 is printed

Consider the following events that happen during a context switch from (user mode of) process P to (user mode of) process Q, triggered by a timer interrupt that occurred when P was executing, in a Unix-like operating system design studied in class. Arrange the events in chronological order, starting from the earliest to the latest. [1.5m]

(A) The CPU program counter moves from the kernel address space of P to the kernel address space of Q.

(B) The CPU executing process P moves from user mode to kernel mode.

(C) The CPU stack pointer moves from the kernel stack of P to the kernel stack of Q.

(D) The CPU program counter moves from the kernel address space of Q to the user address space of Q.

(E) The OS scheduler code is invoked.

- B E C A D

B C E A D

B C E D A

B E C D A

Which of the following actions by a running process will always result in a context switch of the running process, even in a non-preemptive kernel design? [1m]

A. Servicing a disk interrupt, that results in another blocked process being marked as ready/runnable.

B. A blocking system call.

C. The system call exit, to terminate the current process.

D. Servicing a timer interrupt.

Q. The Limited Direct Execution protocol for CPU virtualization involves –

- allowing the process to directly execute non privileged instructions for efficiency

- OS retains control but allows processes to perform certain restricted operations through system calls and dual mode operation

- OS takes certain hardware support to do this efficiently

- no overhead and maximum efficiency

Q. Following are the role(s) of hardware in LDE protocol of CPU virtualization –

- saving and restoring of process context during change of mode (user to kernel and vice versa)

- set and reset mode bit to indicate mode of operation

- store the addresses of system call handlers

- perform the work of system call